VLSM Addressing Scheme

Assumptions- Each subnet/zone with end users has an assumed number of devices. This is used for the calculation to determine the amount of usable host needed for the subnet. To leave room for growth our calculation is: (Assumed Hosts) x 2 = # of usable hosts needed. This calculation accounted for the current number of hosts assumed in the department while allowing for future growth. It is assumed that there are more hosts outside of the ones simulated in our packet tracer network.

| Zone | OSPF Area | VLAN ID | Full Allocation | Subnet Allocation | Assumed Hosts | Zone/Area Reasoning |
| --- | --- | --- | --- | --- | --- | --- |
| Engineering | Area 1 | 10 | 10.1.0.0/22 | 10.1.0.0/24 | 100 | Separates engineering devices to their own area to keep sensitive work protected |
| Corporate | Area 2 | 20 | 10.2.0.0/22 | 10.2.0.0/23 | 200 | Keeps critical corporate documentation separated into its own area. |
| Contractor/Guest | Area 3 | 30 | 10.3.0.0/22 | 10.3.0.0/25 | 50 | Isolates guests to their own area to limit access to network resources |
| Server/DMZ | Area 4 | 40 | 10.4.0.0/22 | 10.4.0.0/28 | 6 | Places servers in a dedicated area to separate server traffic from user network traffic |
| Management backbone | Area 0 | 99 | 10.0.0.0/24 | 10.0.0.0/28 10.0.0.16/28 | 10 | Separates remote management traffic to its own area |
| Inter-zone / backbone links | Area 0 | — | 10.0.255.0/24 | 10.0.255.0/30 10.0.255.4/30 10.0.255.8/30 10.0.255.12/30 10.0.255.16/30 | N/A | Serves as central OSPF backbone to connect all areas |
| Reserved for future zones | Areas 5+ | — | 10.5.0.0/16 | N/A | N/A | Allocated room to add new zones later without changing any addresses. |

Device Addressing

Engineering Zone

| End Device | Interface | IPv4 Address | Default Gateway |
| --- | --- | --- | --- |
| PC0 | NIC | 10.1.0.1 | 10.1.0.254 |
| PC1 | NIC | 10.1.0.2 | 10.1.0.254 |
| Multilayer Switch0 | VLAN 10 | 10.1.0.254 | N/A |

Corporate Zone

| End Device | Interface | IPv4 Address | Default Gateway |
| --- | --- | --- | --- |
| PC2 | NIC | 10.2.0.1 | 10.2.1.254 |
| PC3 | NIC | 10.2.0.2 | 10.2.1.254 |
| Multilayer Switch0 | VLAN 20 | 10.2.1.254 | N/A |

Contractor/Guest Zone

| End Device | Interface | IPv4 Address | Default Gateway |
| --- | --- | --- | --- |
| PC4 | NIC | 10.3.0.1 | 10.3.0.126 |
| PC5 | NIC | 10.3.0.2 | 10.3.0.126 |
| Multilayer Switch0 | VLAN 30 | 10.3.0.126 | N/A |

Server/DMZ Zone

| End Device | Interface | IPv4 Address | Default Gateway |
| --- | --- | --- | --- |
| PC6 | NIC | 10.4.0.1 | 10.4.0.14 |
| PC7 | NIC | 10.4.0.2 | 10.4.0.14 |
| Server1 | NIC | 10.4.0.3 | 10.4.0.14 |
| Server0 | NIC | 10.4.0.4 | 10.4.0.14 |
| Multilayer Switch1 | VLAN 40 | 10.4.0.14 | N/A |

Management backbone Zone

| End Device | Interface | IPv4 Address | Default Gateway |
| --- | --- | --- | --- |
| SW-ENG | VLAN 99 | 10.0.0.2 | 10.0.0.1 |
| SW-CORP | VLAN 99 | 10.0.0.3 | 10.0.0.1 |
| SW-GUEST | VLAN 99 | 10.0.0.4 | 10.0.0.1 |
| SW-DMZ | VLAN 99 | 10.0.0.20 | 10.0.0.17 |
| SW-MGMT | VLAN 99 | 10.0.0. 21 | 10.0.0.17 |
| PC8 | NIC | 10.0.0.18 | 10.0.0.17 |
| PC9 | NIC | 10.0.0.19 | 10.0.0.17 |
| Multilayer Switch0 | VLAN 99 | 10.0.0.1 | N/A |
| Multilayer Switch1 | VLAN 99 | 10.0.0.17 | N/A |

Inter-zone / backbone links

| End Device | Interface | IPv4 Address | Connected too |
| --- | --- | --- | --- |
| Multilayer Switch0 | Gig0/1 | 10.0.255.1 | Router0 |
|  | Gig0/2 | 10.0.255.5 | Router1 |
| Multilayer Switch1 | Gig0/1 | 10.0.255.9 | Router1 |
|  | Gig0/2 | 10.0.255.13 | Router0 |
| Router0 | Gig0/0/0 | 10.0.255.2 | Multilayer Switch0 |
|  | Gig0/0/2 | 10.0.255.14 | Multilayer Switch1 |
|  | Gig0/0/1 | 10.0.255.17 | Router1 |
| Router1 | Gig0/0/0 | 10.0.255.10 | Multilayer Switch1 |
|  | Gig0/0/2 | 10.0.255.6 | Multilayer Switch0 |
|  | Gig0/0/1 | 10.0.255.18 | Router0 |

Router ID Allocation

| End Device | Loopback/Router ID |
| --- | --- |
| Router0 | 10.0.0.33 |
| Router1 | 10.0.0.34 |
| Multilayer Switch0 | 10.0.0.35 |
| Multilayer Switch1 | 10.0.0.36 |

OSPF Area Summary Table

| Device | Area | Network | Subnet Mask |
| --- | --- | --- | --- |
| Multilayer Switch0 | Area 1 | 10.1.0.0 | 255.255.252.0 |
|  | Area 2 | 10.2.0.0 | 255.255.252.0 |
|  | Area 3 | 10.3.0.0 | 255.255.252.0 |
| Multilayer Switch1 | Area 4 | 10.4.0.0 | 255.255.252.0 |

Each zone fits inside its own block, so the rest of the network sees it as just one route.